Profile.txt - Profile mast data

1, 2, 3, 4, 5 refers to the heights with 1 being the highest

% 1 jaar

% 2	DOY doy+dec. hours

% 3	hhmm

% 4	Ubatt

% 5	Ptemp

% 6	rh1

% 7	rh2

% 8	rh3

% 9	rh4

% 10	rh5

% 11	T1 8m

% 12	T2 4m

% 13	T3 2m

% 14	T4 1m

% 15	T5 0.5m

% 16	U1 8m

% 17	dd1 wind direction

% 18	U2 4m

% 19	dd2

% 20	U3 2m

% 21	dd3

% 22	U4 1m

% 23	dd4

% 24	U5 0.5m

% 25	dd5

cnr1raw.txt – individual 10 sec samples of the CNR1 sensor

% "TIMESTAMP","SWTop","SWBottom","LWTop","LWBottom","cnr1_T_K"

% 1 jaar

% 2 doy (incl seconds resolution)

% 3 hhmm

% 4 "SWTop"

% 5 "SWBottom"

% 6 "LWTop" raw data, no Tcnr1 bias correction

% 7 "LWBottom" raw data, no Tcnr1 bias correction

% 8 "cnr1_T_K" (Kelvin)

CNR1.txt – radiation data logged via the large orange logger box

NOTE: column 16 and 17 were additionally included

% CNR1_DATA 1mins averaged  C = 9.97 cal coeffs used inside CAMPBELL logger

% 1 jaar

% 2 doy

% 3 hhmm

% 4 "SWTop_Avg" short wave in

% 5 "SWBottom_Avg" short wave out

% 6 "LWTop_Avg" Longwave in raw

% 7 "LWBottom_Avg" longwave out raw

% 8 "cnr1_T_C_Avg"

% 9 "cnr1_T_K_Avg"

% 10 "LWTopC_Avg" Longwave in

% 11 "LWBottomC_Avg" Longwave out

% 12 "Rs_net_Avg"

% 13 "Rl_net_Avg"

% 14"albedo_Avg"

% 15 "Rn_Avg"

% 16 "LWTopC_Avg_biased" longwave in Tcnr1 biased +3C

% 17 "LWBottomC_Avg_biased" longwave out Tcnr1 biased +3C

iWS.txt column format

A lot of data in here but you only need a few columns/variables

% 1	year

% 2	mm month

% 3	dd day

% 4	hh

% 5 minutes

% 6	doy Day Of Year incl decimal hours/min/sec

% 7	TC1avg	(?C)	-9.96       thermocouple data

% 8	TC1std	(?C)	0.28 thermocouple data standard deviations

% 9	TC2avg	(?C)	-10.31        thermocouple data

% 10	TC2std	(?C)	0.26    thermocouple data standard deviations

% 11	CJavg	(?C)	-10.36

% 12	xTSTR1	(?C)	-10.14

% 13	xTSTR2	(?C)	-10.11

% 14	xTSTR3	(?C)	-10.11

% 15	xTSTR4	(?C)	-10.11

% 16	xTSTR5	(?C)	-10.09

% 17	xTSTR6	(?C)	-10.25

% 18	xTSTR7	(?C)	-7.52

% 19	xTSTR8	(?C)	-4.02

% 20	THUTavg	(?C)	-10.28  raw Tair

% 21	RHWavg	(%)	91.24   raw RHair

% 22	RHTavg	(?C)	-10.44  raw RH sensor temperature data

% 23	HWSavg	(m/s)	6.39    wind speed

% 24	HWSstd	(m/s)	0.78    wind speed standard deviation

% 25	HWSmax	(m/s)	8.47    wind speed max

% 26	HWDavg	(?)	81.65       wind direction

% 27	VWSavg	(m/s)	0.00

% 28	VWSstd	(m/s)	0.00

% 29	NRUavg	(W)	2.4         short wave in

% 30	NRUstd	(W)	0.2

% 31	NRLavg	(W)	3.0         short wave out

% 32	NRIUavg	(W)	263.8        long wave in

% 33	NRILavg	(W)	273.8       long wave out

% 34	NRTavg	(?C)	-10.34  temp from radiation sensor

% 35	NRUcal	(?V)	13.73

% 36	NRLcal	(?V)	13.37

% 37	NRIUcal	(?V)	6.91

% 38	NRILcal	(?V)	8.55

% 39	NRID	(n)	121096

% 40	BAP	(hPa)	794.7      air pressure

% 41	SSH	(m)	3.083          sensor heights / changing snow height

% 42	xADW	(m)	2.332

% 43	xTBRG	(mm)	0.0

% 44	MCH	(?)	342.20

% 45	TILTX	(?)	1.15

% 46	TILTY	(?)	-1.27

% 47	LON	(?)	-470.228.291

% 48	LAT	(?)	670.003.638

% 49	HMSL	(m)	1.848.078

% 50	PACC	(m)	2.43

% 51	SATS	(n)	11

% 52	spare 1

% 53	spare 2

% 54	VBAT	(V)	5.147

% 55	IDENTIFIER card=1, datalogger=2 or ARGOS=3 data (used to be LBUT	(days)  )

% 56	STATUS	(hex)	0

% 57    iwsID

% 58	xID	(n)	58 identifier X is for slave unit data

% 59	xTC1avg	(?C)	326.45

% 60	xTC2avg	(?C)	326.51

% 61	xCJavg	(?C)	-10.44

% 62	xTHUTavg	(?C)	327.67`

% 63	xRHWavg	(%)	100.00

% 64	xRHTavg	(?C)	128.86

% 65	xHWSavg	(m/s)	0.00

% 66	xHWDavg	(?)	0.52

% 67	xVWSavg	(m/s)	0.00

% 68	xSSH	(m)	2.074

% 69	xSPARE1	(-)	12op

% 70	xVBAT	(V)	3.468

% 71	xSTATUS	(hex)	02 station visit indicator !!!

Header for the turbulence data (incl in the file as well). Below I added some comments to some variables

Time

U sonic anemometer

V

W

Ts sonic

Tc thermocouple

Uu covariances

Uv

Uw

uTs

uTc

Vv

Vw

Ww

wTs

wTc

TsTs

TcTc

Uy windspeed from Young horizontal propeller

Wdy wind direction from young propeller

Wg vertical windspeed from propeller

Tc2 thermocouple next to vertical propeller

Uyuy

Uywg

Vyvy

Vywg

Wgwg

wgTc2

Header voor de turb.txt 30mins gemiddelde ruwe turbulentiedata rechtstreeks uit de logger.

Notice that the data within file “turb.txt” is raw datalogger averaged data.

e.g. to obtain u* use uw and vw as in txtbooks

u* = (uw.^2 + vw.^2).^0.25;

% covariances from datalogger 10HZ

First all variables sampled at 10Hz by the sonic anemometer are given

% 1	jaar

% 2	DOY daynumber + decimal hours

% 3 hhmm

% 4	uu

% 5	uv

% 6	uw   covariance of u and w component sonic anemometer

% 7	uTs

% 8 uTc1

% 9 uH2O not available

% 10 vv

% 11 vw

% 12 vTs

% 13 vTc1

% 14 vH2O. not available

% 15 ww

% 16 wTs

% 17 wTc1

% 18 wH2O ( g/m3) not available

% 19 std Ts

% 20 std Tc1

% 21 std H2O not available

% 22 u  average u component

% 23 v

% 24 w

% 25 Ts  sonic temperature

% 26 Tc1 thermocouple temperature close to sonic

% 27 H2O  (g/m3) not available

Below are all variables from the fast sampled horizontal propeller vane, vertical propeller and a thermocouple Tc2 (also at 10Hz)

% 28 u vect average wind speed from Young proppeller vane

% 29 dd vect average wind direction from Young proppeller vane

% 30 std dd

% 31 ww

% 32 wu

% 33 wTc2

% 34 uw

% 35 uTc2

% 36 TcTc2

% 37 w

% 38 u

% 39 Tc2

% 40 dd