Profile.txt - Profile mast data

1, 2, 3, 4, 5 refers to the heights with 1 being the highest

% 1 jaar

% 2	DOY doy+dec. hours

% 3	hhmm

% 4	Ubatt

% 5	Ptemp

% 6	rh1

% 7	rh2

% 8	rh3

% 9	rh4

% 10	rh5

% 11	T1 8m

% 12	T2 4m

% 13	T3 2m

% 14	T4 1m

% 15	T5 0.5m

% 16	U1 8m

% 17	dd1 wind direction

% 18	U2 4m

% 19	dd2

% 20	U3 2m

% 21	dd3

% 22	U4 1m

% 23	dd4

% 24	U5 0.5m

% 25	dd5

cnr1raw.txt – individual 10 sec samples of the CNR1 sensor

% "TIMESTAMP","SWTop","SWBottom","LWTop","LWBottom","cnr1_T_K"

% 1 jaar

% 2 doy (incl seconds resolution)

% 3 hhmm

% 4 "SWTop"

% 5 "SWBottom"

% 6 "LWTop" raw data, no Tcnr1 bias correction

% 7 "LWBottom" raw data, no Tcnr1 bias correction

% 8 "cnr1_T_K" (Kelvin)

CNR1.txt – radiation data logged via the large orange logger box

NOTE: column 16 and 17 were additionally included

% CNR1_DATA 1mins averaged  C = 9.97 cal coeffs used inside CAMPBELL logger

% 1 jaar

% 2 doy

% 3 hhmm

% 4 "SWTop_Avg" short wave in

% 5 "SWBottom_Avg" short wave out

% 6 "LWTop_Avg" Longwave in raw

% 7 "LWBottom_Avg" longwave out raw

% 8 "cnr1_T_C_Avg"

% 9 "cnr1_T_K_Avg"

% 10 "LWTopC_Avg" Longwave in

% 11 "LWBottomC_Avg" Longwave out

% 12 "Rs_net_Avg"

% 13 "Rl_net_Avg"

% 14"albedo_Avg"

% 15 "Rn_Avg"

% 16 "LWTopC_Avg_biased" longwave in Tcnr1 biased +3C

% 17 "LWBottomC_Avg_biased" longwave out Tcnr1 biased +3C

iWS.txt column format

A lot of data in here but you only need a few columns/variables

% 1	year

% 2	mm month

% 3	dd day

% 4	hh

% 5 minutes

% 6	doy Day Of Year incl decimal hours/min/sec

% 7	TC1avg	(?C)	-9.96       thermocouple data

% 8	TC1std	(?C)	0.28 thermocouple data standard deviations

% 9	TC2avg	(?C)	-10.31        thermocouple data

% 10	TC2std	(?C)	0.26    thermocouple data standard deviations

% 11	CJavg	(?C)	-10.36

% 12	xTSTR1	(?C)	-10.14

% 13	xTSTR2	(?C)	-10.11

% 14	xTSTR3	(?C)	-10.11

% 15	xTSTR4	(?C)	-10.11

% 16	xTSTR5	(?C)	-10.09

% 17	xTSTR6	(?C)	-10.25

% 18	xTSTR7	(?C)	-7.52

% 19	xTSTR8	(?C)	-4.02

% 20	THUTavg	(?C)	-10.28  raw Tair

% 21	RHWavg	(%)	91.24   raw RHair

% 22	RHTavg	(?C)	-10.44  raw RH sensor temperature data

% 23	HWSavg	(m/s)	6.39    wind speed

% 24	HWSstd	(m/s)	0.78    wind speed standard deviation

% 25	HWSmax	(m/s)	8.47    wind speed max

% 26	HWDavg	(?)	81.65       wind direction

% 27	VWSavg	(m/s)	0.00

% 28	VWSstd	(m/s)	0.00

% 29	NRUavg	(W)	2.4         short wave in

% 30	NRUstd	(W)	0.2

% 31	NRLavg	(W)	3.0         short wave out

% 32	NRIUavg	(W)	263.8        long wave in

% 33	NRILavg	(W)	273.8       long wave out

% 34	NRTavg	(?C)	-10.34  temp from radiation sensor

% 35	NRUcal	(?V)	13.73

% 36	NRLcal	(?V)	13.37

% 37	NRIUcal	(?V)	6.91

% 38	NRILcal	(?V)	8.55

% 39	NRID	(n)	121096

% 40	BAP	(hPa)	794.7      air pressure

% 41	SSH	(m)	3.083          sensor heights / changing snow height

% 42	xADW	(m)	2.332

% 43	xTBRG	(mm)	0.0

% 44	MCH	(?)	342.20

% 45	TILTX	(?)	1.15

% 46	TILTY	(?)	-1.27

% 47	LON	(?)	-470.228.291

% 48	LAT	(?)	670.003.638

% 49	HMSL	(m)	1.848.078

% 50	PACC	(m)	2.43

% 51	SATS	(n)	11

% 52	spare 1

% 53	spare 2

% 54	VBAT	(V)	5.147

% 55	IDENTIFIER card=1, datalogger=2 or ARGOS=3 data (used to be LBUT	(days)  )

% 56	STATUS	(hex)	0

% 57    iwsID

% 58	xID	(n)	58 identifier X is for slave unit data

% 59	xTC1avg	(?C)	326.45

% 60	xTC2avg	(?C)	326.51

% 61	xCJavg	(?C)	-10.44

% 62	xTHUTavg	(?C)	327.67`

% 63	xRHWavg	(%)	100.00

% 64	xRHTavg	(?C)	128.86

% 65	xHWSavg	(m/s)	0.00

% 66	xHWDavg	(?)	0.52

% 67	xVWSavg	(m/s)	0.00

% 68	xSSH	(m)	2.074

% 69	xSPARE1	(-)	12op

% 70	xVBAT	(V)	3.468

% 71	xSTATUS	(hex)	02 station visit indicator !!!